Article Light engine cor 4 17 04

Light Multi-reflex Engine*

## Alexander Bolonkin

Senior Research Associate of the USA National Research Council at Eglin Air Force Base

1310 Avenue R, #F-6, Brooklyn, NY 11229, USA

e-mail: aBolonkin@juno.com

### Abstract

The purpose of this article is to call attention to the revolutionary idea of multi-reflection. This idea allows the design of new engines, space propulsion systems, storage of a beam and solar energy, transmission of energy over millions of kilometers, a new weapon, etc. This method and its main innovations were offered by the author in 1983 in the former USSR.  Now the author shows in a series of articles the huge possibilities of this idea in many fields such as space, aviation, energy, energy transmission, beam amplification, light transformation and so on. This article considers the direct transfer of light beam energy to mechanical energy and back.

Key words: Bolonkin light engine, multireflex engine, transfer of light to mechanical energy.

*Full text is published in JBIS vol.57, pp.353-359, 2004.

### 1.	Introduction

  - Short history.

In 1972 Kantrowitz called attention to the use of a laser beam for a space propulsion system [1]. He offered the conventional ways: to transfer energy by laser beam to a space apparatus, to convert light energy into heat and evaporate matter, and to get thrust by evaporated matter. There are many researches on this method [2]. However, this method is complex, has low efficiency, has limited range (divergence of the laser beam), requires special matter located on board the space ship, and requires a very powerful laser.

In 1983 the author offered another method of using beam (light) energy: direct conversion of light energy to mechanical pressure (for engine) or thrust (for launchers and propulsion systems) by multiple reflection [3].

The throwing away or reflecting of light is the most efficient method for a propulsion system. It gives the maximum possible specific impulse (speed 3 x 108 m/s). The system does not expend mass. This is important for space ships, because launching mass to space is very expensive (at present time it costs about $20,000-$50,000 /kg). However, the light pressure is very small at about 0.310-6 kg/ kW (the solar light pressure without reflection equals about 0.210-6 kg/m2). In 1983 the author offered the idea to increase the light pressure by multireflex method and he offered some innovations which dramatically decrease the loss of mirror reflection. That allows the system to make some millions of reflections and to get some Newtons of the thrust per one kW of beam power. It allows the design of many important devices (in particular, engines) which transfer light directly to mechanical energy and solve many problems in aviation, space, energy and energy transmission.

The achievements in optic materials and lasers in the last several years have decreased the loss from reflections. The author returned to this topic and made this the primary research. He solved the main problems which appear in research: design of a the very high efficiency reflector, light lock, self-focusing lightweight mirrors and lenses, beam transferor for very long distance (millions of km)  without beam divergence, light storage, beam amplifier, etc. As with any new innovation it appears many additional problems will be need to be addressed in the research and development. However, the solution of these problems promises a revolution in many technical fields.  The author wants to attract the attention of researchers and engineers to these innovative ideas.

  - Brief information about light and used light devices.

The optical diapason of electromagnetic radiation is approximately from a few nanometers to 1 cm. Visible light is approximately located in the interval 400 to 700 nm (1 nm is 10-9 m). Ultraviolet radiation is shorter than visible light while infrared rays are longer than visible light. Microwave radio waves have a higher diapason than the optical diapason, and X-rays have a smaller optical diapason.

The conventional mirror can reflect a maximum of 98-99% light energy of some light waves. It can give a maximum of 200-300 reflections. This is not enough, because the light pressure is very small and equals about 0.6.10-6 kg/ kW (for high reflection). For technical applications it is necessary for a minimum of a million reflections.

The layers of a quarter-wave optical thickness of high- and low-refractive-index materials increase the reflectance. After more than 12 layers, the reflective efficiency of a dielectric mirror approaches about 100% with virtually no absorption or scattering. This reflectance is a maximum only in a region around the design wavelength. The size of the region depends on the design of the stack of multiple dialectric coatings. Outside this region the reflectance is reduced. For example, at one-half the design wavelength it falls to that of the uncoated substrate. The dielectric mirror is also designed to be used at a specific angle of incident radiation. At other angles, the performance is reduced, and the wavelength of maximum reflectance is shifted.

This method is not suitable for our purpose. The wave length is strongly changed for a mobile reflector (rotary disk). The conventional mirror reflects the beam in another direction if mirror plate is not perpendicular to beam.

The narrow laser beam is most suitable for a light engine. There are many different types of lasers with different powers (pick power up 1012 W), wavelength (0.2 to700 μm), efficiency (1% up to about 100%), and impulse (up to some thousands of impulses per second) or continuous operations. A gas laser may be suitable for our purpose. Many molecular gases, such as hydrogen cyanide, carbon monoxide, and carbon dioxide, can provide laser action. Carbon dioxide lasers can be operated at a number of wavelengths near 10 μm on various vibrational-rotational spectral lines of the molecule. They can be relatively efficient, up to about 30%, and have been made large enough to give continuous power outputs exceeding tens of thousands of watts.

At the present time we have seen significant advances in high-power weapons-class lasers. The Missile Defense Agency/USAF Airborne Laser (ABL) program is rapidly approaching first light of the six-module chemical oxygen-iodine laser (COIL) at the heart of the weapon. Preparations are under way at the System Integration Laboratory, for a moth-balled 747-200 converted into a ground test platform, for first light testing of the megawatt-class COIL flight hardware. The laser mirror is larger than 1.5 m in diameter.

The Air Force Research laboratory (AFRL) has contracted with Northrop Grumman Space Technologies and Raytheon to develop a high-power solid-state 25-kW laser.

The laser beam divergence equals

θ=2πλD=1.13λD	,					(1.1)

where  θ is the angle of divergence [rad],  λ is the wavelength [m], and D is the aperture diameter [m]. Diffraction theory gives the coefficient in (1.1) equal to 1.22 .

The solar light has a radiation spectrum approximately in the interval λ=0.4 to 2.2 μm and maximum energy at λ=0.5 μm. The divergence of solar light is about 0.006 radian.

More detailed information is given in [1,2,4-9].

2.	Description of innovation

For getting maximum reflectance degree 450 prisms 1 (fig.1a) are used. For incident angles greater than sin-1(n1/n2), no light is transmitted. Here n is the refractive index (n ≈ 1 to 4). Instead the light is totally reflected back into the incident medium. This effect is called total internal reflection. Total internal reflection is used for our reflector. Our reflector contains two plates having a set of small prisms reflecting a beam from one prism to another prism (fig.1b). Every plate can contain millions of small (30 to 100 μm) prisms. (The optical cable uses a cable diameter of 2-10 micron). The entered beam 3 is reflected millions of times as shown in fig. 1c,d (see a path 2) and creates a repulsive force F. This force may be very high, thousands N/kW (see computation below) for motionless plates. It is limited only by absorption (dB) of the prism material (see below).

The prisms and their cells can have a slope (fig.2a) and located on a rotor outer surface and a stator inter surface (fig.2b) or plates (fig.2a). This system can be applied to rotary or linear engines. Two tasks appear: how the beam enters inside this cell (without possibility of leaving the cell) and how much force the system has? The prism material attenuates the light energy and decreases the beam

power (wave amplitude).

For solution of the first problem the author offers “a light lock” which allows the light beam to enter but closes the exit (fig. 1b). Fig.1 shows three designs of the light locks.

Fig.1. Multireflection system: a) Prism with full internal reflection: 1 – prism, 2 – path of a light beam, 10 – gap between the prism and body of plate, rotor, stator (it may be filled by a matter (cover) with n1 < n2 , n – reflective index; b) Pass  of beam through series of prisms: 3 – entrance beam, 4 – entrance canal, 5 – mirror (light lock – version one); 6 – light lock (version 2)(multilayer mirror) , F – repulsive force; c) The first version of the beam path through the reflected cells; d) The second version of  the beam path through the reflected cells; e) Light lock (version 2): 6 - multi layer mirror, 7 – entrance beam, 8 – beam after worked path (λ8≠λ7); f) The light lock (version 3): 9 – additional prism, 10 – variable gap between main prism 1 and additional prism 10.

The first light lock is shown in fig. 1b. The beam inters narrow beam canal 4. The entering beam runs the full path 2 (fig. 1b,c,d), reflects in millions of cells, and comes up to enter the cell from the other side. Here the beam is partially reflected back into the system by mirror 5 and partially reflected and repeated the old way in a straight direction. In mobile plates part of the beam is reflected back to the laser and the laser mirror sends it again to the system. This part of the beam equals s1/s2, where s1 is the width of the beam canal and s2 is the length of a prism hypotenuse.

The second light lock is shown in fig. 1e. It has the mirror 6 with multi-layer coating. This coating passes the entered beam 3 (7), but it well reflected the interned beam 8. It is possible because the wavelengths of beams 7 and 8 are different (λ8≠λ7) in mobile plates.

The third light lock is shown in fig. 1f. It contains the additional prism 9 and used an impulse laser. When the laser beam 3 inters into the system, the prism, 9, is pressed to main prism 1. While the beam runs the path 2, the additional prism 10 is disconnected from prism 1 and the beam 3 can only travel inside the reflected system while it expends all energy. The chink 10 may be very small, about a light wavelength (1 micron). A piezoelectric plate can be used for this purpose.

Fig.2. Sloping reflective cells and light engine: a) 11 – stator (motionless), 12 – mobile rotor, 13 – local force (of single cell), 14 – direction of rotor movement; b) Light engine: 20 – rotor, 21 – stator.

A pulsed or continuous laser may be used. We compute an average laser power.

The detailed attenuation of light propagating through optical matter is considered in a theoretical chapter. For increasing the number of reflection we use a set of very small prisms and current high efficiency optical matter.

3. 	Theory of multireflex engine

The attenuation of light propagating through optical matter is caused either by absorption or by scattering. In both absorption and scattering, the power lost over a distance, z, and the power N(z), propagating at that point is given by an exponential decay:

N(z)=N(0)exp(-γz) .						(3,1)

The attenuation coefficient, γ, is normally expressed in dB km-1, with 1 dB km-1 being the equivalent of 2.3 ×10-4 m-1. Absorption is a material property in which the optical energy is normally converted into heat. In scattering processes, some of the optical power in the guided modes is radiated out of the material.

Attenuation of some current and some possible very low loss materials created for fiber communication is presented in fig. 3. However, some of these materials are highly reactive chemically and are mechanically unsuitable for drawing into fibers. Some are used as infrared light guides. None is presently used for optical communication but may be used for our purposes. Our mechanical property and wavelength requirements are less than in optical communication. However, we use in our computation conventional values of 0.1 to 0.4 dB. Clean air has ξ =0.333×10-6 m-1. The conventional optical matter widely produced currently in industry has an attenuation coefficient equals to 2 dB.

Fig.3. The estimation of basic attenuation of some possible very low loss materials.

The beam power will be decreased if one (or both) reflector is moved, because the wavelength is increased. The total loss of the beam energy in one double cycle (light beam is moved to reflector and back) is

q = (1-2γ)(1-2ξ)(1±2v),						(3.2)

where v=V/c,  V is relative speed of cells [m/sec], c = 3×108 m/s is light speed. We take the “+” when the distance is decreased (braking) and take “-“ when distance is increased (usefull work by light).

The light pressure, T, of two opposed high reflectors after a series of reflections, n, to one another is

T0=2N0c , T1=2N0c q , T2=2N0c q2 , T3=2N0c q3, ... , Tn−1=2N0c qn , 	(3.3)

This is a geometric series. The sum of n members of the geometric series is

T=2N0c qn−1q−1 . If n=∞ then T∞=2N0c 11−q , q<1.			(3.4)

If the reflector is moved off, the maximum number of reflections, n, is limited by cell size, l, because the wavelength is increased and that cannot be more than the cell size. This limit is

n≤ln(lλ0)2v	,						(3.5)

where l is cell width [m], λo is the initial wavelength [m].

If the reflector is moved forward to another, the wave length is decreased, but it cannot be less than the length of X-rays because the material transmits the X-rays (material lowers the reflective capability).

This limit is about λmin=10 nanometers. Maximum number of brake reflections is

n≤ln(λ0λmin)2v .						(3.6)

The force of the engine with slope, α, cells (fig.2) and engine coefficient efficiency, η, may be computed by the equation

T=2N0c qn−1q−1 cosα , η=TVN0 .					(3.6)

The values in equation (3.2) can be computed by

γ=0.00023al , l=mλ , m≥1 , ξ=0333⋅10−6lc , lc=l+δtanα ,		(3.7)

where a is the attenuation coefficient in dB [km] (see fig. 3), m is the initial number of wavelengths which can be located in sell size l, lc is the length of a cell [m], and δ is the gap between engine disks [m].

The engine rotor force coefficient, A, shows how many times the initial light pressure is increased. It is

A=qn−1q−1 .						(3.8)

The results of computations are presented in figs. 4- 8.  The computed parameters are not optimal. Our purpose is demonstrating the method of computation.

Fig.4 shows the maximum pressure which can be created by a one kW beam in the cell design of fig.1b for a motionless disk (V=0, a = 0.1 dB, m = 30, N0=1 kW, λ= 0.1 to 1 μm). This pressure reaches a high force of 5000 N for the wavelength of 1 micron. This high pressure may be used, for example, for moving a heavy device a micron distance, for loud speaker devices, for frequency modulation of laser beam by mechanical energy, etc.

Fig. 5 presents the number of reflections for disk speed V=200 m/s, a = 0.2 dB, m=10 to 300. This number reaches 4 million double reflections (one time from each mirror).

Fig.4 (left). Repulsive force, T, of motionless plates (V=0 m/s) for infinity reflections, attenuation coefficient a = 0.1 dB, cell size m = 30, beam power N0=1 kW and different wavelength.

Fig.5 (right). Force coefficient, A (Eq. (3.8)) and number of reflections n (Eq. (3.5)) for rotor speed V=200 m/s,

a= 0.2 dB, and relative size of prisms m=10÷300.

Fig. 6 presents a transmission efficiency coefficient of light energy, ηt, transferred to mechanical energy for V=200 m/s, a=0.2 dB, λ=0.2, 0.5, 0.7, 1, 2 μm, m=10 to 300. The efficiency of the proposed light engine is very high and reaches 98% – 99% if m>50 (Eq.(3.6)).

Fig. 7 presents a force on the rotary disk of the light engine for different speeds V=20 to 500 m/s, a=0.1 dB, m=30, P=1 kW. This force changes from 50 N through 2 N.

Fig. 8 presents the efficiency coefficient for a rotary engine for the data of fig. 7, when the engine works in a vacuum (-) and in air (- -). The gap between disks is 0.1 mm. This coefficient reaches more than 0.96 and only considers the transformation loss without mechanical friction. The mechanical friction is conventional for turbines and it is small.

All computations are made for the beam power N0=1 kW. For beam power N0=10, 100, 1000 kW we must multiply the forces in fig.7 by 10, 100, 1000 respectively.

We neglect the loss in sides of the cells because the loss is small if the beam divergence is small and a lateral side of cells has the full reflection cover (as in the light cable).

Fig.6 (left). Efficiency coefficient (Eq.(3.6)) for rotor speed V=200 m/s, a=0.2, wavelength λ=0.2 to 2 microns, relative size of prisms m=10 to 300.

Fig.7 (right). Thrust (force) (Eq. (3.6)) versus the rotor speed (V=20 to500 m/s) for wavelength λ=1μm, a=0.1 dB, m = 30, beam power N0=1 kW.

Fig. 8.  Efficiency coefficient (Eq. (3.9)) versus wavelength λ=0.2 to 1 μm, a=0.5 dB, m=30, N0=1 kW, and rotor speed V=100 to 500 m/s. The lines”- -“ is computation with air attenuation.

4. Discussion

The offered multireflex light engine is very simple (only two disks), has a small size, very high ratio of power/weight, and efficiency. One can directly transfer the beam light to mechanical energy.

This engine may be applied in aviation because the energy can be transferred from an earth surface station to aircraft by the laser beam. The aircraft does not carry fuel and has a more lightweight engine. Its load capability increases by two times. Industry can produce a one Megawatt (1000 kW) laser now. This is enough for middle class aircraft with weight at 10 to 12 tons [10].

The linear light engine does not have a limit of speed and one may be used for the launch of space equipment and for space ships [11 - 13]. This method may be also used for the design of new efficient weapons [14].

The intensity of solar light on a clear day is about 1 kW/m2. If we can transfer this light to mechanical energy, the mechanical energy can be transferred to electrical energy. The efficiency of this process may be higher and cheaper than the efficiency of the expensive commercial solar cells (10-12% efficient).

# 5.	Acknowledgement

The author wishes to acknowledge scientist Dr. Don Grundel from the Air Force Research Laboratory AFRL/MNG (Eglin AFB, Florida, USA) for his help in editing and correcting my English.

References

- A. Kantrowitz, “Propulsion to Orbit by Ground-Based Lasers”, Astronautics and Aeronautics, 10, pp.74-76, May 1972.

- “Beamed Energy Propulsion. First International Symposium on Beamed Energy Propulsion”, American Institute of Physics, Melville, New York, 2003.

- A. A. Bolonkin, “Light Pressure Engine”,  Patent (Author Certificate)  # 1183421, 1985, USSR, (priority on January 5, 1983).

- R. G. Driggers (Ed.), “Encyclopedia of Optical Engineering”, Marcel Dekker, Inc., New York, 2003.

- “Handbook of Optics”, Second Edition, McGraw-Hill, Inc., 2000.

- “Military Standardization Handbook. Optical Design”, No. 141, 1962.

- P. W. Milonni, and J. H. Eberly, “Lasers”, John Wiley & Sons., N.Y, 1988.

- D. C. O’Shea, “Elements of Modern Optical Design”, J. Wiley and Sons, N.Y., 1985.

- A. W. Snyder, “Optical  Waveguide Theory”, Chapman and Hall, N.Y.-London, 1983.

- A. A. Bolonkin, “Multi-reflex Propulsion Systems for Space and Air Vehicles and Energy Transfer for Long Distance”, (under publication).

- A. A. Bolonkin, “Kinetic Space Towers and Launchers”, JBIS, 57, pp.33-39, 2004.

- A. A. Bolonkin, “Hypersonic Launch System of Capability up 500 tons per day and Delivery Cost $1 per Lb” IAC-02-S.P.15, 53rd International Astronautical Congress - The World Space Congress, Houston, Texas, USA, 10-19 Oct 2002.

- A. A. Bolonkin, “Earth Accelerator for Space Ships and Missiles”, JBIS, 56, pp. 394-404, 2003.

- A. A. Bolonkin, “Air Cable Transport”, Journal of Aircraft, 40, No.2, March-April, 2003. Patent US 6,494,143 B1.