Article Light Propulsion cor Dave with figs 3 7 04

Multi-reflex Propulsion Systems for Space and Air Vehicles and

Energy Transfer for Long Distance*

Alexander Bolonkin

Senior Researcher of the USA National Research Council at Eglin Air Force Base

1310 Avenue R, #F-6, Brooklyn, NY 11229, USA

Tel/Fax  718-339-4563, aBolonkin@juno.com, http://Bolonkin.narod.ru

Abstract

The purpose of this article is to call attention to the revolutionary idea of light multi-reflection. This idea allows the design of new engines, space and air propulsion systems, storage (of a beam and solar energy), transmitters of energy (to millions of kilometers), creation of new weapons, etc. This method and the main innovations were offered by the author in 1983 in the former USSR.  Now the author shows in a series of articles the immense possibilities of this idea in many fields of engineering - astronautics, aviation, energy, optics, direct converter of light (laser beam) energy to mechanical energy (light engine), to name a few. This article considers the multi-reflex propulsion systems for space and air vehicles and energy transmitter for long distances in space.

Key words: Bolonkin Space Propulsion System, Light Engine, Multi-reflex propulsion, Space Energy transfer.

*Full version is published in JBIS vol.57, pp.379-390, 2004.

1. Introduction

1.1. Short history.  The relatively conventional way to send a spacecraft on an interstellar journey is to use the solar sail [1] or a laser sail [2].  This method is not effective because the light pressure is very small, with only one reflection. There has been a lot of research in this area and of solar sails in general.

A. Kantrowitz offered the conventional method to using a laser beam for space propulsion [3].  He transfer of energy by laser beam to a space vehicle, convert light energy into heat and evaporate a material, then to get thrust from the gas pressure of this evaporated material. There is much research on this method [4]. However, this method is complex, has low efficiency, has limited range (divergence of the laser beam), requires special material located on board the space ship, and requires a very powerful laser.

In 1983 the author offered another method: of using light beam energy, then the direct conversion of light energy to mechanical pressure (for engine) or thrust (for launchers and propulsion systems) by multiple reflections [5].

The author found only one work related to this topic, reference [6] published in 2001.  However our work is very different from [6]. Our suggested system has several innovations (inventions) which makes possible the offered method and improves its parameters millions of times. The difference between our suggested system and the system [6] is analyzed in the “Discussing” section, below.

The reflecting of light is the most efficient method for a propulsion system. It gives the maximum possible specific impulse (light speed is 3.108 m/s). The system does not expend mass. However, the light pressure fol full reflection  is very small, about 0.610-6 kg/ kW.  In 1983 the author suggested the idea to increasing the light pressure by multi-reflex method (multiple reflection of the light beam) and he offered some innovations to dramatically decrease the losses in mirror reflection (include cell mirror and reflection by a super conducting material). This allows the system to make some millions of reflections and to get some Newtons of the thrust per kW of beam power. It allows for the design of many important devices (in particular, beam engines [7]) which transfer light directly to mechanical energy and solve many problems in aviation, space, energy and energy transmission.

In the last years achievements in optic materials and lasers have decreased the losses from reflection. The author returned to this topic and made it his primary area of research. He solved the main problems: the design of a the very high efficiency reflector (cell mirror), a light lock,  focusing prismatic lightweight mirrors and lenses, laser ring, beam transfer for very long distances (millions km) with only very small beam divergence, light storage, beam amplifier, modulator of light frequency, balloon suspending of mirrors, and so on.

1.2. Brief information about light and used light devices.  The short description of electromagnetic radiation the reader find in [7].  A conventional mirror can reflect a maximum of 98-99% of the incident light energy of some band of light waves. This gives a maximum of 200-300 reflections. It is not enough for propulsions and engines. Because the light pressure is so very small (about 0.6.10-6 kg/ kW ), we need at least a million reflections.

There is a well-known method increasing of mirror reflection. The layers of a quarter-wave optical thickness of high- and low-refractive-index materials increase the reflectance. After more than 12 layers, the reflective efficiency of a dielectric mirror approaches about 100% with virtually no absorption or scattering. This reflectance is a maximum only in a region around the design wavelength. The size of the region depends on the design of the stack of multiple dielectric coatings. Outside this region the reflectance is reduced. For example, at one-half the design wavelength it falls to that of the uncoated substrate. The dielectric mirror is also designed for use at a specific angle of incident radiation. At other angles, the performance is reduced, and the wavelength of maximum reflectance is shifted.

Unfortunately, this dielectric mirror method is not suitable for mirrors moving relative to each other as the reflected frequency is shifted slightly, and this frequency shift accumulates over multiple reflections.  Also conventional mirrors tend to reflect the beam off in some other direction if the mirrors are not kept in perfect alignment to the beam. The offered author cell mirror reflects the beam in same direction. It is very important for decreasing of the beam divergence. The small cells provide a high reflectance and small absorption.

A narrow laser beam is the most suitable for a light engine and light propulsion. There are many different types of lasers with different powers (peak power up to 1012 W), wavelength (0.2-700 μm), efficiency (1% up to about 95%), and pulse rate (up to some thousands of impulses per second) or continuous operations. The reader will find a brief description the laser in [7] or more detail in [8].

At present the time we have seen significant advances in high-power weapons-class lasers (see [7]). The laser power reaches one million watts.

For our computation very important the beam divergence. The laser beam divergence equals [8] p.4

θ=2πλD=1.13λD	,			(1.1)

where  θ is angle of divergence [rad],  λ is wavelength [m], and D is an aperture diameter [m]. In particularly, the diameter of the laser beam may be increased by an optical lens for decreasing the beam divergence.  The aperture diameter may be also increased by offered laser ring (fig.2). The reflex capacity may be improved by using a super conductive material [5](this idea needs additional research).

More detailed information is in the references [7]-[9].

2. Description of innovation

2.1. Multi-reflex launch installation of space vehicle.  In multiple reflection propulsion system a set of tasks appear: how to increase a mirror reflective, how to decrease light dispersion (from mirror imperfection and non parallel surfaces), how to decrease a beam divergence, how to inject the beam between the mirrors (while keeping the light between the mirrors for as long as possible), how to decrease the attenuation (a mirror, prism material, etc), how to increase the beam range, and how much force the system has?

For a solution of these problems, the author offers [5], a special “cell mirror” which is very reflective and reflects light in same direction from which it came, a “laser ring” which decreases the beam divergence, “light locks” which allows the light beam to enter but keeps it from exiting, a “beam transfer”, a “focusing prismatic thin lens“, prisms, a set of lens, mirrors located in space, on asteroids, moons, satellites, and so on.

The first peculiarity: Cell mirrors. For getting the maximum reflectance, decreasing light absorption, and saving beam direction author uses a special cell mirrors which have millions of small 450 degree prisms (1 in fig.1a,g). Mirror cell is retroreflector cell or cube corner cell. A light ray incident upon it is returned parallel to itself after three reflections (fig.1g). The mirror provided the refractive index of the prism is greater than 2(1.414), the light will be reflected by total internal reflection. The small losses may be only from a prism (medium) attenuation, scattering, due to small surface imperfections and Fresnel reflections at the entrance and exit faces. Fresnel reflection do not give losses when the beam is perpendicular to enter surface. No enter losses of beam polarized parallel of enter surface or the enter surface having anti-reflection coating with reflective indexn1=n0n2. Here n0, n2 are reflective indexes of vacuum and prism respectively. These cell mirrors turn exactly back a beam (light) at 1800 degree if a beam deviation is less 5-100 from a perpendicular to a mirror surface. For incident angles greater than sin-1(n1/n2), no light is transmitted, an effect called total internal reflection.  Here n is refractive index of medium and lens (n ≈ 1 - 4).  Total internal reflection is used for our reflector which contains two plates (mirrors) having a set of small corner cube prisms reflecting the beam from one side (mirror) to the other side (mirror) (fig.1b,c, f). Every plate can contain millions of small (30 - 100 μm) prisms from highly efficient optic material used the optical cables [9]. For this purpose may be used also a superconductivity mirror [5].

The second peculiarity: Laser ring. The small lasers are located in a round ring (fig.2c).   A round set of lasers allows us to increase the aperture resulting in a smaller divergence angle θ. The entering round beam (9 in fig.1a) has slip θ (or θ/2) to the vertical. The beam is reflected millions of times as it is shown in fig.1b,c  and creates a repulsive force F. This force may be very high, tens of N/kW (see computation below) for motionless plates. In a vacuum it is limited only by the absorption (dB) of the prism material (see below) and beam divergence.  For the mobile mirror (as for a launch vehicle) the wavelength increases and beam energy decreases as the mirrors move apart.

This system [5] can be applied to a space vehicle launch at a planet that has no atmosphere and small gravity (for example, as on the Moon), (high gravity requests a high beam power).

Fig.1 (left). Space Launcher. Notations are: 1- prism, 2 - mirror base, 3 - laser beam, 4 – mirror after chink (optional), 5 –Space vehicle, 6 – lasers (ring set of lasers), 7 – vehicle (ship)  mirror, 8 – planet mirror, 9 –laser beam, 10 - multi-layer dielectric  mirror, 11 – laser beam after multi-reflection (the wavelength λ11>λ9 ), 12 – additional prism, 13 - enter beam, 14 – back beam, 15 – variable chink between main and additional prisms.

(a) Prism (cell, corner cube) reflector. (b) Beam multi-reflection, (c) Launching by multi-reflection, (d) The first design of the light lock, (e) the second design of the light lock, (f) The reflection in same direction when the beam is not perpendicular to mirror surface, (g) Mirror cell (retroreflector cell or cube corner cell). A light ray incident upon it is returned parallel to itself after three reflections.

Fig.2 (right), Laser beam long distance transfer. Notations are: 12 – lens, 14 – bounds of laser rays, 15 - light receiver, 16 – divergence ray.

(a) focused beam, (b) focused beam with angle θ. It has a part S without divergence, (c) focused beam with angle 0.5θ. It has a minimum divergence at a long distance, (d) beam with a plate wave front, (e) Gaussian beam with normal distribution of beam front, (f) Fresnel’s (prism) lens, (g) lens for changing of beam direction.

The third peculiarity: Light lock. The first design of light lock allows the laser beam to enter, but closes the exit of a returned ray. The beam (9 in fig.1d) of continuous laser passes through a multi-layer dielectric mirror (10 in fig.1d). The entering beam runs the full length between mirrors (fig. 1b,c), reflects a million times, and enters from other side (11 in fig.1d).  For moving (separating) mirrors the wavelength is changed because the beam gives up energy to the moving mirrors (see computations). As the result the wavelength increases (λ11>λ9) when distance increases and the wavelength decreases (λ11<λ9) when distance decreases. The mirror (10 in fig.1d), is design to pass the laser beam (9 in fig.1d) and to reflect back the ‘used’ ray (11 in fig.1d). If the beam is not reflected by the mirror (10 in fig.1d), it enters into the laser and will be reflected back by the laser’s internal mirror.

The second design of the light lock is shown in fig.1e.  It contains the additional prism 12 and impulse laser. When laser beam 13 enters into system, the additional prism 12 is pressed to main prism 1. While the beam runs between mirrors, the additional prism is disconnected from main prism and the back beam 14 cannot enter back. That travels inside the reflected mirrors a lot of reflections if the mirrors have right focuses. The chink, 15, between additional and main prisms may be very small, about a light wavelength (1 micron). A piezoelectric plate can be used for moving of the additional prism.

A continuous or pulse laser may be used for the first light lock and the pulse laser may be used for the second lock. We compute an average laser power.

The details of attenuation of light propagating through an optical material is considered in physics textbooks.  To increase the number of reflections, we use a set of very small prisms and a highly efficient optic material (dB=0.1-0.5)(see [7], fig.3).

For a solution of other problems, the author offers: a high efficiency beam  transfer, a beam engine [7], a “focusing beam prismatic thin lens“, thin Fresnel’s prisms, a set of lens, thin mirrors located in space, on asteroids, moons, satellites, and so on.

2.2. Space beam transfer.  Space beam transfer is shown in fig.2a. The first lens gives the large aperture for laser beam and focuses the beam.  That decreases the divergence angle θ.  The other Fresnel’s lens continues to focus the beam (fig.2a).

Non-focused beam losses by diffracted rays.  The beam  transfer has the special focusing lens. If the focus is located in distance S1=D/2θ, the beam does not less the diffracted rays in this distance (S), but  after a distance S gets the divergence angle 2θ (fig.2b). If we need to transmit energy a distance L less than S (for example, in launching), this method is fine since the distance between mirrors L<<S and beam reflects many times without a less. If we want to transfer the energy a very long distance, the method shown in fig. 2c may be better.  In this method the beam is focused to point at a distance S2=D/θ. The beam has a lesser diffraction everywhere, but this lesser amount is smaller after distance 1.5S1 than in the case of fig. 2b. If an intermediate lens having a much greater diameter then initial lens (fig.2b,c) is added midway, it is possible to decreases the beam diffraction energy losses to a very small value.

The distribution of energy in a gross section area of beam is also important for the divergence and diffraction losses. The plate front (fig.2a) of the wave and plate distribution of energy and divergence (fig.2d) is worst and gives a maximum of energy losses. The normal distribution of beam energy and Gaussian beam is better because the diffraction losses of beam energy is lessened at its edges (fig.2e).

The energy transport is done the following way. The Fresnel’s lens (collimators)(fig.2f), Fresnel’s prisms (fig.2g), and mirrors are advance (permanently) located in space (fig.3a). Their trajectories and receiving space vehicle’ trajectory in space are known. By command from Earth, a space ship or the vehicle’ computer, the mirrors and lenses are turned the required angles (angular position). A small pilot ray may be used for aiming and focusing. The requested angular changes are small (for focusing and small correction in direction) and may be made by piezoelectric controlled plates. After this ray reaches the space vehicle properly, the full power beam is transmitted to space vehicle. This beam may be used for vehicle launching from an asteroid or small mass planetary satellites (fig,1c), or for changes to the vehicle trajectory (fig, 3b), or for acceleration of the space vehicle near an asteroid (fig.3c) by multi-reflex method (fig.3a,b,c). This beam energy may be also used by the space vehicle for rocket engine and internal power requirements.  The distance between lenses may reach tens of millions of kilometers (see computation below, fig.13).  It remains the average distances of the nearest planets from Sun are: Venus 108 x106 km, Earth 150 x106 km, Mars 228 x106 km. Transfer efficiency of system may be about 0.7 - 0.9 (see computation below).

2.3. Aviation and energy air transfer.  This method may be used for the transfer of energy to flight, ground, sea, and ocean vehicles (fig. 4). The transfer beam passes an atmosphere only one time and losses a few energy. The vehicle may to use the light (multi-reflex) engines are described in [5],[7]. The multi-reflex engine transfers (converges) the beam (light) energy directly to mechanical energy (rotate propeller) with high efficiency.

Fig.3 (left). Space energy transfer for long distance. Notations are: 20 – Sun, 21 – Earth, 22 – laser beam, 23 - Fresnel’s lens, 24 – mirror, 26 – Fresnel’s prism, 28 – Space vehicle, 30 – planet, 32,34 – planet satellite, 36 – multi-reflection.

Fig.4 (right). Beam energy transfer to air, ground, and sea vehicles. Notations are: 50 – beam energy station, 52 – control mirror suspended to a tether air balloon, 54 – mirror suspended to planet satellite, 56 – aircraft, 58 – ground track, 60 – car, 62 – sea (ocean) ship, 64 - beams.

The system should include: beam energy station(s), control mirrors suspended by air balloons at altitude 12 - 18 km (or on satellites), and a control system. The energy station sends the beam to the nearest mirror. The mirror reflects it to other mirrors (or lens, or prism). These mirrors distribute the beam to customers (aircraft, ground vehicles, river or sea ships, and so on).  The customers may have the light (multi-reflex) engines [5],[7] and directly convert the light (electromagnetic radiation) to mechanical work.  The control system guides the energy distribution.  In a clear atmosphere, the efficiency of this energy delivery system is high. It is important that the air be clean and clear at initial, mostly vertical distance (from energy station to the first mirror).  At times of rain or smoke (from fires, etc.) a backup method should be in place, perhaps from orbiting satellite mirrors.  At altitude over 12 km the atmosphere is conventionally clear.

This method allows us to delivery energy at some thousands of kilometers (see computation below, fig.14).  In war time the beam may be focused and used against the enemy.

3. Theory (estimation) of multi-reflex launching and beam transfer.

Special theory, methods and computation for this case are developed below.

3.1. Attenuation of beam. The attenuation of light propagating through an optical material is caused either by absorption or by scattering. In both absorption and scattering, the power lost over a distance, z, and the power N(z), propagating at that point. So we expect an exponential decay:

N(z)=N(0)exp(-yz) .			  		(3.1)

The attenuation coefficient, y, is normally expressed in dB km-1, with 1 dB km-1 being the equivalent of 2.3×10-4 m-1.  Absorption is a material property in which the optical energy is normally converted into heat.  In scattering processes, some of the optical power in the guided modes is radiated out of the material.

Attenuation of some current and some possible very low loss material created for fiber communication has dB up to a = 0.0001 [7], fig.3.  However, some of these materials are highly reactive chemically and are mechanically unsuitable for drawing into a fiber. Some are used as infrared light guides, none are presently used for optical communication, but may be useful for our purposes.  Our mechanical property and wavelength requirements are less stringent than for optical communications.  We use in our computation  a=0.1 ÷ 0.4 dB km-1. The conventional optical material widely produced by industry for optical cable has the attenuation coefficient equals 2 dB  km-1.

3.2. Change of beam power. The beam power will be decreased if one (or both) reflector is moved, because the wavelength is changed. The total relative loss of the beam energy in one double cycle (light ray is moved to reflector and back) is

q = (1-2γ)(1-2ξ)(1±2v)ς ,					(3.2)

where v=V/c,  V is relative speed of mirrors [m/sec], c = 3×108 m/s is light speed. We take the “+” when the distance is decreased (braking) and take “-“ when distance is increased (as in launching, a useful work for light), γ is the loss from prism attenuation, ξ is loss (attenuation) in medium (air) (clean air has ξ =0.333×10-6 m-1), v is loss (useful work) from relative mirror (lens) movement,  ς is loss from divergence and diffraction .

3.3. Multi-reflex light pressure. The light pressure, T, of two opposed high reflectors after a series of reflections, n, to one another is

T0=2N0c , T1=2N0c q , T2=2N0c q2 , T3=2N0c q3, ... , Tn−1=2N0c qn . 	(3.3)

When q=const, this is a geometric series. The sum of n members of the geometric series is

T=2N0c qn−1q−1 . If n=∞ , then T∞=2N0c 11−q , q<1.		 (3.4)

3.4. Limitation of reflection number. If the reflector is moved off, the maximum number of reflections, n, is limited by cell size, l, because the wavelength, λ, is increased and that cannot be more than the cell size. This limit is

n≤ln(lλ0)2v						(3.5)

where l is cell width [m], λo is the initial wavelength [m].

If the reflector is moved forward to another, the wavelength is decreased, but it cannot be less than the length of X-rays because the material transmits the X-rays (material lessens the reflective capability).

This limit is about λmin=10 nanometers. Maximum number of brake reflections is

n≤ln(λ0λmin)2v .					(3.6)

3.5. Coefficient of efficiency. The efficiency coefficient, η, may be computed by equation

η=TVN0 ,					(3.7)

3.6. Focusing of beam. If it is used the lens focused in range S1, the distance, S, without ray divergence is (fig.2b):

S=D2θ , θ=2πλD , S=π4D2λ=0.443D2λ .			(3.8)

Here, D is diameter of the lens or mirror [m]. This distance equals the lens focus distance for case fig.2b (S1=S). In case fig,2c (transfer for very long distance), the optimal focus distance is equals S2=2S1.

3.7. Some computation. The computation of equation (3.8) is presented in figs. 5-6. As you see, the need focus distance may be high.

The values in equation (3.2) can be computed as

γ=yz=0.00023al , l=mλ , m≥1 , ξ=0333⋅10−6L , 	(3.9)

where a is the attenuation coefficient in dB [km-1] (see [7], fig.3), m is initial number of wavelength which can be located in cell size l [m].

The loss from divergence, ς, for case fig.2b,d equals

ς=π(D2)2π(D2+2γ(L−S))2 , 2γ(L−S)=4kπλ(L−S)D , ς=11+8kλ(L−S)πD22for L>S. (3.10)

Here L is distance between mirrors (lens) [m].  And, k is focus coefficient: in case fig.2b (focus distance equals D/2θ) k = 0 when L<S (for transfer), or  n<S/L (for reflection)  and k = 2 when L>S, or n>S/L; in case fig.2c (S is absent, S=0) k = 0.5 if focus distance equals D/θ; k=1 if focus distance is infinity (no focusing).

Relative beam power along its trajectory for plate power distribution of fig.2d is

N=N/N0=1 while L≤S1 and N=ς when L>S1=D2θ.	         (3.11)

The force coefficient, A, shows what times the initial light pressure is increased. For L<S1 it is

A=qn−1q−1 .					         (3.12)

Fig.5 (left). Focus distances [ 106  km, million km] versus lens diameters 1-10 m and the wave  λ=0.2-1

microns.

Fig.6 (right). Focus distances versus lens diameters D=1-100 m and the wavelength λ=0.2-1 microns.

The multi-reflex launch of a space vehicle from a small planet with low gravity, without an atmosphere (the Moon or an asteroid) may be computed by the following equations (for focusing fig.2b and beam distribution fig.2d):

T=2N0cqn1−1q−1+2N0cqn1q1n2−1q1−1 , n1=S1L , n2=n1−n3 , n3=lnm2v , q=(1−2γ)(1−2v) , q1=qς, ΔV=TM−gΔt , Vi+1=Vii+ΔV, ΔL=ViΔt . Li+1=Li+ΔL , ti+1=ti+Δt .  (3.13)

Here: The first member in T is the trust when the beam runs the distance S1 without divergence.

Fig.7 (left). Trust (N) versus distance [m] for vehicle speed V=10 - 200 m/s, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m, beam power N=1 kW.

Fig.8 (right). Efficiency coefficient versus distance [m] for vehicle speed V=10 - 200 m/s, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m, beam power N=1 kW.

The second member in T is the thrust when beam runs the distance with divergence. M is space vehicle mass [kg], g is the planet’s gravity [m/s2].  When n3 < n1, we take n=n3 and compute T by equation (3.4).  If n3 > n1, we compute T by equation (3.13).

Computation of trust, T (the first eq.(3.13)) and efficiency coefficient, η, (eq. (3.7)) are presented in figs.7-8.

The trust and efficiency coefficient decrease when distance is over some critical value, then a portion of the energy beam leaves the space between the mirrors by diffraction.

The computation of a vertical (the worst case) launch (Eq.(3.13)) of a space vehicle from the Earth’s Moon (the Moon has gravity g=1.62 m/s2) are presented in fig.9-12 (λ=1 micron).

The required power to launch from the Earth’s Moon is quite high because, as the vehicle’s speed increases, energy transfer efficiency is reduced.  In the initial launch state that efficiency starts at η=0.97.  After an acceleration (limit is 16g, g=9.81 m/s) for a distance of L=2 - 10 km the efficiency decreases (fig.10).  The acceleration is high only 0.1-2 sec (16g is admissible for trained cosmonauts). The mission designers could limit the maximum acceleration to 8g without significantly losing speed.

The mirror diameter is large. Small mirror diameters decrease the attainable speed. Starting from an asteroid or a planet’s moon, having a small gravity improves the attainable speed.  Unfortunately, the multi-reflex launch from planets with an atmosphere is poor because the multi-reflected rays run long distances in a gas medium and losses a lot of energy.

Below is the equation for computing the beam power from the divergence and distance when the Gaussian beam has normal distribution (fig.2f): Case #1 (focus is in point 2S1, fig.2c)

N1=2ψsDD+θL2 , θ=2πλD , S=0.	                  (3.14)

Here ψ if probability function of normal distribution.

Case #2 (focus located in point S, fig.2b)

While L≤S1, N2=1. When L>S1 N2=2ψsDD+4θ(L−S1)2.     (3.15)

Fig.9(left). Vertical launching of a space vehicle from the Earth’s Moon (Moon gravity is g =1.62 m/s2).  Space ship speed (m/s) versus distance (km) is computed for relative beam power N=5-50 kW/kg, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m. Acceleration is limited by 16g (g=9.81 m/s2).

Fig.10 (right). Vertical launching of a space vehicle from the Earth’s Moon (Moon gravity is g =1.62 m/s2). Efficiency coefficient versus distance is computed for relative beam power N=5-50 kW/kg, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m.

Fig.11 (left). Vertical launching of a space vehicle from the Earth’s Moon (Moon gravity is g =1.62 m/s2). Space ship speed (m/s) versus time (sec) is computed for relative beam power N=5-50 kW/kg, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m, λ=1 micron.

Fig.12 (right). Vertical launching of a space vehicle from the Earth’s Moon (Moon gravity is g =1.62 m/s2). Limited (<16g=16x9.81=157 m/s2) acceleration is computed for relative beam power N=5-50 kW/kg, attenuation coefficient a=0.5 dB, cell size m=30, mirror diameter D=100 m, λ=1 micron.

Case #2 (focus located in point S, fig.2b)

While L≤S1, N2=1. When L>S1 N2=2ψsDD+4θ(L−S1)2.     (3.15)

Here s is a relative distribution value. The results of computations for space (vacuum) are presented in fig. 13. It is shown that the focused beam is transferred without major losses if the distance between the mirrors (for mirror diameter D=100-200 m) is 10 -18 millions kilometers, and may be transferred up 100 millions km with an efficiency of about 0.2 .  This means the focused beam can permanently transfer (without losses) energy from Earth to Moon or back (distance 0.4 x106  km), and for 2-3 months (with efficiency 0.2) every two years, to Mars at distance 60 - 150 x106  km.

For the computation of the relative beam power in air at altitude H, we may use equations (3.14)-(3.15) corrected for air attenuation. That is

Na1=N1(1−b) , Na2=N2(1−b) , where b=0.334⋅10−6ρHρ0L .      	         (3.16)

Here ρH, ρo are air density at altitude H and H=0 respectively.

The computation for an atmospheric balloon lens located at H=15 km (ρH/ρo=0.159) is presented in fig.14.  As can be seen, the beam energy may be transferred by mirrors located at an air balloon to distances 4000 km out, with efficiency 0.8 . If the energy is transferred through a satellite the range is not limited and efficiency may reach 0.98 .  Any smog, mist, haze, rain, clouds (in the region of the power station) decreases the efficiency.  However, these aerosols are absent at altitudes above 12 km. We must have power stations in different locations to circumvent weather phenomena.

Fig.13(left). Relative beam power of the normal (Gaussian) distribution (s=σ=2.5) (fig.2f) in vacuum versus distance in million kilometers between lens for focusing D/2θ (- - , case #1) and D/θ (-, case #2).

Fig.14(right). Relative beam power of the normal (Gaussian) distribution (s=σ=2.5) (fig.2f) in air (altitude H=15 km) versus distance in kilometers between lens for focusing D/2θ (- -) and focusing D/θ (-).

The computed parameters are not optimal. Our purpose is demonstrating the method of computation.

Computations (fig.7-8) are made for the beam power N0=1 kW. For beam power N0=10, 100, 1000 kW we must multiply the force figs.7-8 by 10, 100, 1000 respectively.

5. Discussing

5.1. Compare the method “Multi-Bounce Laser-Based Sail” [6] and the offered method “Multi-Reflex Propulsion System”.

1. “Multi-Bounce Sail” uses the well known multi-layer mirror. That has high reflectance ONLY in a region about the design wavelength. Outside this region the reflectance is reduced.  For example, at one-half the design wavelength it falls to that of the uncoated substrate.  As shown in this work, the wavelength is changed a small amount at each reflection at the mobile mirror. This means after enough reflections the multi-layer mirror has lost the high reflective. It is impossible to use the multi-layer mirror for multi-bounce space sail that is moving. The author offered the innovative new CELL - MIRROR. It's reflectivity does NOT depends on wavelength for wavelengths less then a cell length.

2. The multi-layer mirror [6] has gigantic size (1 km), extremely small thickness (1600 nm), density (10 gm/m2) and weight (7850 kg).  A very small angle of a deviation of the multi-layer mirror surface (one thousandth of degree) under beam pressure, makes for a full defocusing at a distance of some millions of kilometers.  This means the mirror [6] will make only ONE reflection. The average mirror angle will also be changed permanently for moving space ship.

It is impossible to exactly control (turn) the pointing of this gigantic and very thin sail.

The new CELL – MIRROR reflects the laser beam in back in EXACTLY the same direction if the surface and sail deviation is less 5-10 degree. This means the mirror pointing control is not necessary, on the space craft. This means the film surface imperfections may be and the mirror control is not necessary.

3. The maximum reflection of multi-layer mirror is 99.95 [6]. The reflection of the cell mirror is (1-0.4.10-9) or 108 times better than the multi-layer mirror. The maximum reflection number of the multi-layer mirror is only 1000 [6]. The reflections of the cell mirror are some millions.

4. The diameter of the multi-layer mirror [6] is 1 km, the size of our cell –mirror is 100 m.

5. The gigantic multi-layer mirror [6] gives an acceleration of only 0.33 m/s2. This is not enough to launch itself from Earth (Earth’s gravity is 9.8 m/s2), Mars (3.72 m/s2) or the Moon (1.62 m/s2).  The author's cell-mirror gives an acceleration 20 m/s2, and its size is 100 times smaller!!  If we made cell-mirror 1 km in diameter, the capability of a space ship will be fantastic.

The author shows only some advantages of one innovation (change of the well-known multi-layer mirror to the new cell-mirror). There are a lot of deficiencies of the System in reference [6] which make all but impossible in its application.  For example, in [6] the laser is located on Earth’s surface and its beam moves (from the laser to ship and back to the laser) across the Earth’s atmosphere a lot of times. The computation shows that the beams energy will quickly be lost to absorption and scattering by Earth’s (or Mars) atmosphere when it runs a long distance though it.  In our system the beam moves across the atmosphere only ONE time and reflects between Moon mirror and space ship all other times. This is insured by the innovation of the light lock.

Another deficiency of System [40]: when the space ship is closed to Earth, the sail will reflect the beam back to laser. If the efficiency of the propulsion system were adequate, the laser might be damaged or destroyed. This problem is absent in the authors system because it uses the "light lock", which closes the return path of laser beam.

The suggested laser ring (set of small lasers located in circle), beam transfer and self-focused mirror and Fresnel’s  lens decrease the beam divergence and increase a beam transfer distance.  It is possible to install the cell-mirror at the Moon or Mars and transfer a laser beam to them and then to make a space ship decelerates.

The system [6] requires the building a nuclear electric power station (of several Giga Watts!) and to deliver it and a super powerful laser to Mars.

I do not mean to criticize other small mistakes in the work [6] as, for example, the computation of multiple reflection acceleration (thrust) is not correct.  The beam energy after every reflection will be decreased and the ships acceleration also will be decreased.  For a large number of the reflections this decrease is quite large (see equations in this article).

The idea multiple reflection engine and cell and superconductivity mirrors was probability offered at first by author on 1983 [5]. But as I know, the work [6](2001) was the first research of this topic. That is important.

5.2. General discussing. The offered multi-reflex light launcher, space and air focused energy transfer system is very simple (needing only special mirrors, lens, prisms), and it has a high efficiency.  One can directly transfer the light beam to space acceleration and mechanical energy. A distant propulsion system can get its energy from the Earth.  However, we need very power lasers.  Sooner or later the industry will create these powerful lasers (and cell mirror) and the ideas presented here will become possible. The research on these problems should be started now.

Multi-reflex engines [7] may be used in aviation as the energy can be transferred from the power stations on the ground, to the aircraft by laser beams. The aircraft would no longer carry fuel and the engine is lighter weight.  Its load capability would double. The industry produces a one Megawatt (1000 kW) laser now.  This is the right size for mid-weight aircraft (10-12 tons).

The linear light engine does not have a limit of speed and one may be used for the launch of space equipment and for space ships in non-rockets methods described in [10]-[29].  This method is certain to have many military applications.

Acknowledgement

The author wishes to acknowledge scientist David Handwerk, a graduate student from the University of Southern California - Aerospace Engineering Department, for his help in editing and correcting my English and asking questions to aid in its clarification.

References

- R.L. Garwin, "Solar Sailing - A practical Method of Propulsion within the Solar System," Jet Propulsion, Vol 28, March 1958, pp. 188-190.

- R.L. Forward, "Pluto - The Gateway to the Stars," Missiles and Rockets, Vol 10, April 1962, pp. 70-75.

- A. Kantrowitz, Propulsion to Orbit by Ground-Based Lasers, Astronautics and Aeronautics, V.10, 74-76, (May, 1972).

- Beamed Energy Propulsion. First International Symposium on Beamed Energy Propulsion. Melville, New York, 2003. American Institute of Physics.

- A.A. Bolonkin, Light Pressure Engine.  Patent (Author Certificate)  # 1183421, 1985, USSR, (priority on January 5, 1983).

- R.A.Metzger and G.Landis, Multi-Bounce Laser-Based Sails, Proceedins of the shape Technology and Application, Forum-2001, Albuquerque, New Mexico, 11-14, 2001, AIP Conference Proceedings, Vol.552, pg.397 [File: STAIF 2001 B].

- A.A. Bolonkin, Multreflex Engine, (under publication).

- P.W. Milonni, J.H. Eberly, Lasers, John Wiley & Sons., N.Y.

- R.G. Driggers, editor, Encyclopedia of Optical Engineering., New York, Marcel Dekker, Inc.

- A.A. Bolonkin, Earth Accelerator for Space Ships and Missiles. JBIS, v.56, No. 11/12, pp. 394-404.

- A.A. Bolonkin, Kinetic Space Towers and Launchers, JBIS, Vol.57, No.1/2, pp.33-39, 2004.

- A.A. Bolonkin, Bolonkin Anti-gravitator, AEAT, #3, 2004.

- A.A. Bolonkin, Air Cable Transport, Journal of Aircraft, Vol.40, No.2, March-April, 2003. Patent US 6,494,143 B1.

- A.A. Bolonkin,“Non-Rocket Space Rope Launcher for People”, IAC-02-V.P.06, 53rd International Astronautical Congress. The World Space Congress – 2002, 10-19 Oct 2002/Houston, Texas, USA.

- A.A,  Bolonkin, “Non-Rocket Missile Rope Launcher”, IAC-02-IAA.S.P.14, 53rd International   Astronautical Congress. The World Space Congress – 2002, 10-19 Oct 2002/Houston, Texas, USA.

- A.A. Bolonkin,  Optimal trajectory of air vehicles, AEAT, #2, 2004.

- A.A. Bolonkin, , “Inexpensive Cable Space Launcher of High Capability”, IAC-02-V.P.07, 53rd International Astronautical Congress. The World Space Congress – 2002, 10-19 Oct.    2002/Houston, Texas, USA.

- A.A. Bolonkin, “Employment Asteroids for Movement of Space Ship and Probes”. IAC-02-S.6.04, 53rd International Astronautical Congress. The World Space Congress – 2002, 10-19 Oct. 2002/Houston, Texas, USA.

- A.A. Bolonkin, “Non-Rocket Earth-Moon Transport System”, COSPAR-02 B0.3-F3.3-0032-02, 02-A-02226, 34th Scientific Assembly of the Committee on Space Research (COSPAR). The World Space Congress – 2002, 10-19 Oct 2002/Houston, Texas, USA.

- A.A. Bolonkin, “Non-Rocket Earth-Mars Transport System”, COSPAR-02 B0.4-C3.4-0036-02, 34th Scientific Assembly of the Committee on Space Research (COSPAR). The World Space Congress – 2002, 10-19 Oct 2002/Houston, Texas, USA.

- A.A. Bolonkin,  “Transport System for Delivery Tourists at Altitude 140 km”. IAC-02-IAA.1.3.03, 53rd International Astronautical Congress. The World Space Congress – 2002, 10-19 Oct. 2002/Houston, Texas, USA.

- A.A. Bolonkin, Asteroids as Propulsion Systems of Space Ships, JBIS, Vol.58, pp.97-107, 2003.

- A.A. Bolonkin, Non-Rocket Transportation System for Space Travel, JBIS, Vol.56, No 7/8, pp. 231-249, 2003.

- A.A. Bolonkin, Centrifugal Keeper for Space Stations and Satellites, JBIS, Vol.56,  pp.314-327, 2003.

- A.A. Bolonkin, Non-Rocket Earth-Moon Transport System, Advances in Space Research, Vol.31/11, pp.2485-2490, 2003, Elsevier.

- A.A. Bolonkin, Earth Accelerator for Space Ships and Missiles. JBIS, v.56, No. 11/12, pp. 394-404.

- A.A. Bolonkin, Kinetic Aviation, AEAT, #3, 2004.

- A.A. Bolonkin, Long Distance Transfer of Mechanical Energy, (under publication).

- A.A. Bolonkin, Non-Rocket Transport System Earth-Mars, Actual problems of aviation and aerospace systems, Kazan, Russia, No.2(16), vol.8, 2003.